1. Volodymyr Shcherban’  -  Doctor of Technical Sciences, Professor

2. Oksana Kolisko  -  Ph.D., Associate Professor

3. Gennadiy Melnyk - Ph.D., Associate Professor

4. Marijna Goldberg  -  Ph.D., Associate Professor

5. Anton Kirichenko  -  Doctor of Philosophy, Аssistant

5. Valerij Kalashnyk - Ph.D. , Аssistant

In accordance with the agreement and full payment ($ 30) for an article «INFLUENCE OF THE DEFORMATION OF THE THREAD SECTION IN THE DIRECT CONTACT ZONE ON THE VALUE OF ACCELERATION OF THE CENTRAL AXIS POINTS» V. Scherban, O. Kolisko, G. Melnyk, M. Goldberg, A. Kirichenko, V. Kalashnyk   in your log, please send electronic version of the magazine our article on the electronic address

scherbanvu@ukr.net

Sincerely authors.

INFLUENCE OF THE DEFORMATION OF THE THREAD SECTION IN THE DIRECT CONTACT ZONE ON THE VALUE OF ACCELERATION OF THE CENTRAL AXIS POINTS

Volodymyr Shcherban’

Doctor of Technical Sciences, Professor, Department of Mechanical Engineering,

Kyiv National University of Technologies and Desing, Ukraine

ORCID ID: 0000-0002-4274-4425

e-mail: scherbanvu@ukr.net

Oksana Kolisko

-  Ph.D., Associate Professor,  Department of Computer Science, Kyiv National University of Technologies and Desing, Ukraine

ORCID ID: 0000-0003-4043-1238

Gennadiy Melnyk

Ph.D., Associate Professor,  Department of Computer Science, Kyiv National University of Technologies and Desing, Ukraine

ORCID ID: 0000-0002-0002-7663

Marijna Goldberg

Ph.D., Associate Professor,  Department of Computer Science, Kyiv National University of Technologies and Desing, Ukraine

ORCID ID: 0000-0002-9982-7264

Anton Kirichenko

Doctor of Philosophy,  Department of Computer Science, Kyiv National University of Technologies and Desing, Ukraine

ORCID ID: 0000-0003-0041-3799

Valerij Kalashnyk

Ph.D., Department of Computer Science, Kyiv National University of Technologies and Desing, Ukraine

ORCID ID:0000-0002-2345-4377

Abstract

Determined projections of the acceleration vector in relation to the arc length coordinate of the thread axis through components that define the degree of thread compression in the contact zone. Solving the challenges faced by the textile and light industries in increasing the production of fabrics and knitted goods based on improving the productivity of existing equipment and employing modern technologies, grounded in the latest scientific achievements, has significant economic and technical importance. The number of breakages can be significantly reduced by using high-quality raw materials and optimizing the conditions of thread processing on technological equipment, where interaction with guiding and working elements of large and small curvature occurs. Theoretical foundations for the accelerations of points on the thread axis, considering compression during its movement along a guide of large curvature, have been developed.

Keywords: acceleration, deformation, thread cross-section, guiding surface, central axis.

Introduction.  The improvement of many technological processes in the textile and light industries** should be based on theoretical and experimental studies of the interaction of threads with the working elements of technological equipment [1, 2, 4]. By working elements, we refer not only to needles, plates, thread guides, combs on knitting machines, and heald frame holes, and reed teeth on weaving looms, but also to various types of thread guides, thread tensioning devices, and tension compensators [3-6]. The process of forming a fabric and knitted element does not end at the last attached stitch or knitted loop; it continues as the fabric moves away from the formation zone or the return plane for knitting [8, 12]. This is explained by the interaction between threads, where stabilization of the thread position in the fabric and knit will occur when the equilibrium condition between the individual force elements, components of the fabric and knit in that area, is met [11]. Hence, studying the interaction of threads in the working zone will complement the comprehensive research of the thread processing process on textile and light industry technological equipment. Theoretical research should contribute to a broader understanding of the processes of thread interaction with guiding surfaces of large curvature, considering the compression of both the threads and the guides, which will help optimize these processes in terms of reducing breakage [4, 6-8]. The stiffness in bending and compression in the contact zone with the guide depends on the amount of thread twist. High-twist threads and yarns have high bending stiffness and low deformation in the contact zone, while low-twist threads and yarns have low bending stiffness and high deformation in the contact zone. Considering the speed of modern knitting and textile equipment, it has been proven that the acceleration of the change in the position of the thread axis points in the contact zone with the guide of large curvature significantly affects the thread tension after the guiding surface [4-9]. The dependencies of the thread axis points' speed on time and arc length coordinate are used to determine the acceleration of the thread axis points.

The purpose of the article.  Determine the effect of cross-sectional deformation of the thread in the contact zone with the guide on the magnitude of the acceleration of the central axis points.

Results.

The fundamental differential equation for the change in velocity as a function of the arc length coordinate is as follows

|  |  | (1) |
| --- | --- | --- |

where ω0​ is the angular velocity of rotation of the principal trihedron τ0n0b0​ (without considering deformation); ωu ​ is the angular velocity of rotation of the principal trihedron caused by the movement of the thread axis points due to the deformation of the thread in the contact zone with the guiding surface.

Equating the right sides of the system of equations describing the distribution of velocities of the points on the axis of the deformed thread and the vector equality (1), in projection onto the axes of the principal trihedron τ*n*b*, we obtain

|  |  | (2) |
| --- | --- | --- |

where ω0b, ωub are the projections of the angular velocity vectors ω0,  ωu onto the binormal of the principal trihedron; ω0n, ωun are the projections of the angular velocity vectors of the rotating thread element onto the normal axis.

The system of equations (2) is used to determine the projections of the acceleration vector onto the coordinate axes τ*,n*, b*

|  |  | (3) |
| --- | --- | --- |

where W*ф, W*n, W*b - projections of the acceleration vector onto the axes of the principal trihedron.

To determine the relationship between the projections of the acceleration vector and the arc length coordinate, we differentiate expression (3) with respect to S and obtain

|  |  | (4) |
| --- | --- | --- |

where the projections of the first derivative of the acceleration vector of point А* with respect to the arc length coordinate onto the axes of the principal trihedron.

The first derivative of the acceleration vector with respect to the arc length coordinate can be represented as

|  |  | (5) |
| --- | --- | --- |

Considering expression (1)

|  |  | (6) |
| --- | --- | --- |

where е1, е2, е3 - accordingly to the projections of the angular acceleration vector onto the axes ф*,n*, b*, transform formula (5).

|  |  | (7) |
| --- | --- | --- |

Equating the corresponding projections of vector equation (7) to the right sides of system (4), we obtain

The system of equations (3) can be expressed in expanded form through the components that define the degree of thread compression in the contact zone. Taking into account equations (5)–(7), system (4) will take the form

|  |  | (8) |
| --- | --- | --- |

If the thread moves in a plane (for example, τ*,A*, n*), then system of equations (8) will be transformed into

|  |  | (9) |
| --- | --- | --- |

In the case of rectilinear motion of the thread, system (9) will transform into a system of identities.

Typically, in calculations, longitudinal shear during compression in the contact zone is neglected, and only the transverse component is considered. In this case, system of equations (8) simplifies to

Conclusions. Determined the effect of cross-sectional deformation of the thread in the contact zone with the guide on the magnitude of the acceleration of the central axis points.

References

1. Shcherban’ V., Makarenko J., Petko A., Melnyk G., Shcherban’ Yu., Shchutska G. Computer implementation of a recursion algorithm for determining the tension of a thread on technological equipment based on the derived mathematical dependences // Eastern-European Journal of Enterprise Technologies. -    2020. - volume 104. -№2/1. – pp.41-50.

2. Scherban V. Equalizations of dynamics of filament interactive with surface/V. Scherban, G. Melnik, A.Kirichenko,  O. Kolisko, M. Sheludko//Intellectual Archive, Toronto: Shiny World Corp., Richmond Hill, Ontario, Canada. – January/February 2017. – Volume 6.- Number 1. – P. 22-26.

3. Shcherban V., Kolva N., Egorov D., Petko A.,  Makarenko Ju.  Algorithmic and Computer Software for Determination of Thread Tension After Guide Large Curvacity // Intellectual Archive, Toronto: Shiny World Corp., Richmond Hill, Ontario, Canada. – January - March 2021. – Volume 10.- Number 1. – P. 15-20.

4. Щербань В.Ю. САПР оборудования и технологических процессов легкой и текстильной промышленности/В.Ю.Щербань, О.И.Волков, Ю.Ю.Щербань. – К.:Бумсервис, 2004. – 519 с.

5. Scherban V. Basic parameters of curvature and torsion of the deformable thread in contact with runner/V. Scherban, N.Murza, A.Kirichenko, O. Kolisko, M. Sholudko //Intellectual Archive, Toronto: Shiny World Corp., Richmond Hill, Ontario, Canada. – Nov/Des - 2016. – Volume 10.- Number 2. – P. 18-23.

6. Shcherban’ V. Yarn tension while knitting textile fabric/V.Shcherban’ , G. Melnyk , M.Sholudko , O.Kolysko, V.Kalashnyk// Fibres and Textiles. – 2018. - volume 25. - №3. - pp. 74-83.

7. Vasilchenko V.N.,  Shcherban V.Yu. Influence of the twist of a capron complex filament on the value of its flexural rigidity// Technology of the textile industry. - 1986. - №4. - P.8-9.

8. Scherban V.Yu. Investigation of the process of duck surf during the formation of multilayer technical fabric// Technology of the textile industry. - 1990. - №4. - P.41-44.

9. Scherban V.Yu. Determination of the geometric characteristics of the shape of the filament axis moving along the deformable guide surface // Technology of the textile industry. - 1990. - №6. - P.52-55.

10. Shcherban V.Y. Basic design support of CAD in the fashion industry/VY Shcherban, YY Shcherban, OZ Kolisko, GV Melnik, MI Sholudko, VY Kalashnik. - K.: Education of Ukraine, 2018. – 902 р.

11. Scherban V. Yu. Mathematical Models in CAD. Selected sections and examples of application/V. Yu. Scherban, SM Krasnitsky, VG Rezanovа. - K.: KNUTD, 2011. – 240 р.

12. Shcherban’ V.Improvement of structure and technology of manufacture of multilayer technical fabric/V.Shcherban’ , G. Melnyk , M.Sholudko , O.Kolysko, V.Kalashnyk// Fibres and Textiles. – 2019. - volume 26 - № 2 - pp. 54-63.